Step 1 – Get a copy of the Teensy program file Download 4Rose.hex to your computer.

Step 2 – Get a copy of the Nextion configuration file Download RoseEngine_14.tft, and put it in the root folder of a microSD card. Notes: - You can only have one file on this card which is a .TFT file. The upgrade will not work if there is a 2d one. - You need to check for hidden files on the microSD card. I found that hard to do on macOS (which created a hidden TFT file) and had to do this on a Raspberry Pi.

Step 3 – Get a copy of the 4Axes.ini settings file Download 4Axes.ini, and put it in the root of a second microSD card

Step 4 - Upgrade the Teensy Step 4a Connect the computer to the Teensy. - The cable end connected to the Teensy uses a MicroUSB connector. - The Teensy’s jack is noted in the picture to the right. Notes: - Be sure you have a good MicroUSB cable. Some are only used for charging a device (and they are usually longer). These will often not work.

Step 4b On the Computer, start the Teensy Loader. The icon looks like the one to the right. Note: - If you don’t have the Teensy Loader, you can get it from https://www.pjrc.com/teensy/loader.html.

Step 4c Be sure the system shows that the Teensy is connected. This is evident when the Auto button is illuminated green. As noted in step 4a above, if your USB cable is bad, this won’t light. The default for the CPU selected won’t handle the whole 4Rose.hex file, so if you try to open it, you will get the message as shown here.

Step 4d Press the button on the Teensy to load the get the Teensy loader to recognize which CPU you are using.

Step 4e Back on the Computer, using the Teensy Loader, open 4Rose.hex. Once this is loaded, the bottom message will change to reflect the file size. (this picture was not updated).

Step 4f Press the button on the Teensy to load the program file. Disconnect the computer from the Teensy, and then reboot the Teensy.

Step 5 – Upgrade the Nextion Secure the microSD card which has the Nextion configuration file (from step 2). Put this microSD card into the Nextion’s microSD card slot. Reboot the Teensy again. If this works as expected, it will read Check Data … 100% Updated Successed! (guess that is a bad translation from Chinese). After that message appear, remove the microSD card from the Nextion, and reboot the Teensy again.

Step 6 - Load the .ini file Secure the microSD card which has the 4Axes.ini settings file (from step 3). Put this microSD card into the Teensy’s microSD card slot. Start the system and click the Load ini button. The Load ini button will turn green, and then it will take off. Once completed, a message will appear saying Done – Restart may be needed. Reboot the system one last time.

Step 7 – Verify the Settings It's a good practice to verify all of the settings once you've completed the steps. - Touch the Config button on each page and verify the Teensy column is populated with the same numbers as the Nextion column. Repeat for each axis on each page. - Next verify the Preferences are correct for the Spindle, Axes, Limits, and Returns. - Select the Main page, then the '...' (Preferences) page. Select the More page. Touch EEPROM. Repeat for each page and axis on the page. This will show all of the settings for the selected page. A copy of the settings file (4Axes.ini) is on the following sheets.

Config file settings

;====================================

[Setup]

;====================================

BoardType=4

Microsteps_Spindle=32

StepsPer360_Spindle=200

GearRatio_Spindle=9

Polarity_Spindle=1

;Z Axis

Microsteps_Z=32

StepsPer360_Z=200

DistancePer360_Z=.02

Polarity_Z=1

;X Axis

Microsteps_X=32

StepsPer360_X=200

DistancePer360_X=.02

Polarity_X=1

;B Axis

Microsteps_B=32

StepsPer360_B=200

GearRatio_B=150

Polarity_B=1

DistancePer360_B=1.02

RadialOrLineal=0

BRadius=5

;====================================

[Limits]

;====================================

;Limit Switches

Min_Z=34

Max_Z=35

Min_X=36

Max_X=37

Min_B=38

Max_B=39

;====================================

[Returns]

;====================================

;Return

MaxSpeed_Spindle=15011

Accel_Spindle=15012

MaxSpeed_Axis_Z=15013

Accel_Axis_Z=15014

MaxSpeed_Axis_X=15015

Accel_Axis_X=15016

MaxSpeed_Axis_B=15017

Accel_Axis_B=15018

;====================================

[Main]

;====================================

;Axis Ids:

;-----------------

;Z=0

;X=1

;B=2

;Spindle=3

;-----------------

AxisId=0

;Spindle

MaxSpeed_Spindle=15000

Accel_Spindle=15001

SpeedPercentage_Spindle=30

;Z Axis

MaxSpeed_Z=4001

Accel_Z=5002

SpeedPercentage_Z=53

;X Axis

MaxSpeed_X=4000

Accel_X=5005

SpeedPercentage_X=10

;B Axis

MaxSpeed_B=5007

Accel_B=5008

SpeedPercentage_B=59

;====================================

[One]

;====================================

AxisId=3

;Spindle

MaxSpeed_Spindle=25060

Accel_Spindle=20061

SpeedPercentage_Spindle=61

;Z Axis

MaxSpeed_Z=5062

Accel_Z=5063

SpeedPercentage_Z=64

;X Axis

MaxSpeed_X=5065

Accel_X=5066

SpeedPercentage_X=67

;B Axis

MaxSpeed_B=5068

Accel_B=5069

SpeedPercentage_B=70

;====================================

[Ind]

;====================================

IndexId=1

MaxSpeed_Spindle=5070

Accel_Spindle=5071

SpeedPercentage_Spindle=72

;-----------------

;Division:0  Degrees:2

;Fixed:0  File:1

;-----------------

;Index 1

DivisionsOrDegrees_1=2

FixedOrFile_1=0

Size_1=1.875

;Index 2

DivisionsOrDegrees_2=2

FixedOrFile_2=0

Size_2=24.375

;Index 3

DivisionsOrDegrees_3=2

FixedOrFile_3=0

Size_3=90

;====================================

[Mov]

;====================================

AxisId=0

;Z Axis

MaxSpeed_Z=5080

Accel_Z=5081

SpeedPercentage_Z=82

Distance_Z=3

;X Axis

MaxSpeed_X=5083

Accel_X=5084

SpeedPercentage_X=85

Distance_X=5.75

;B Axis

MaxSpeed_B=5086

Accel_B=5087

SpeedPercentage_B=88

Distance_B=5.78

;====================================

[BE]

;====================================

AxisId=0

;Spindle

MaxSpeed_Spindle=5090

Accel_Spindle=5091

SpeedPercentage_Spindle=92

;Z Axis

MaxSpeed_Z=5093

Accel_Z=5094

SpeedPercentage_Z=95

;X Axis

MaxSpeed_X=5096

Accel_X=5097

SpeedPercentage_X=98

;B Axis

MaxSpeed_B=5099

Accel_B=5100

SpeedPercentage_B=50

;====================================

[Sync]

;====================================

AxisId=0

HelixType=0

Revolutions=.3

Distance=10

;Spindle

MaxSpeed_Spindle=5040

Accel_Spindle=5041

SpeedPercentage_Spindle=42

;Z Axis

MaxSpeed_Z=5043

Accel_Z=5044

SpeedPercentage_Z=45

;X Axis

MaxSpeed_X=5046

Accel_X=5047

SpeedPercentage_X=48

;B Axis

MaxSpeed_B=5048

Accel_B=5049

SpeedPercentage_B=49

;====================================

[Rec]

;====================================

AxisId=0

RadialOrAxial=0

;Spindle

MaxSpeed_Spindle=5030

Accel_Spindle=5031

SpeedPercentage_Spindle=32

;Z Axis

MaxSpeed_Z=5033

Accel_Z=5034

SpeedPercentage_Z=35

;X Axis

MaxSpeed_X=5036

Accel_X=5037

SpeedPercentage_X=38

;B Axis

MaxSpeed_B=5039

Accel_B=5040

SpeedPercentage_B=39

;Radial

Radial_Waves=5

Radial_Spindle_Amplitude=20

Radial_Axis_Distance=5

;Axial

Axial_Waves=4

Axial_Spindle_Degrees=25

Axial_Axis_Amplitude=7

;====================================

[Grk]

;====================================

AxisId=0

FileOrPattern=0

;Spindle

MaxSpeed_Spindle=5020

Accel_Spindle=5021

SpeedPercentage_Spindle=22

;Z Axis

MaxSpeed_Z=5023

Accel_Z=5024

SpeedPercentage_Z=25

;X Axis

MaxSpeed_X=5026

Accel_X=5027

SpeedPercentage_X=28

;B Axis

MaxSpeed_B=5028

Accel_B=5029

SpeedPercentage_B=29

;Pattern page

RadialOrAxial_Pattern=0

;4a: 2 4b: 3 3a: 6 3b: 7 2a: 4 2b: 5

PatternType=4

Pattern_PatternsPer360=8

Pattern_PatternCount=1

Pattern_SegmentLength=1

;File page

RadialOrAxial_File=0

File_PatternsPer360=7

File_PatternCount=1

File_SegmentLength=1

;Segments: 2  Actual: 3

File_SegmentsOrActual=2

;====================================

[Geo]

;====================================

AxisId=0

RadialOrAxial=0

Rose_n=7

Rose_d=5

;Spindle

MaxSpeed_Spindle=5010

Accel_Spindle=5011

SpeedPercentage_Spindle=12

;Z Axis

MaxSpeed_Z=5013

Accel_Z=5014

SpeedPercentage_Z=15

RadialAmplitude_Z=2

AxialAmplitude_Z=2.5

;X Axis

MaxSpeed_X=5016

Accel_X=5017

SpeedPercentage_X=18

RadialAmplitude_X=2

AxialAmplitude_X=2.5

;B Axis

MaxSpeed_B=5018

Accel_B=5019

SpeedPercentage_B=19

RadialAmplitude_B=3

AxialAmplitude_B=3.5